- Consider a hemispherical surfacez(x,y)=(r2−x2−y2)+z0.  Select an image of size 64x64. With the origin as the centre, and r=24 (say), find the surface normal representing the gradients p(x,y) and q(x,y) analytically. Assume the surface to be Lambertian. The object is illuminated by a point light source with(ps,qs)=(0,0). Generate the corresponding image.

- Use regularization based approach in (p,q) and (f,g) domains to recover the shape. Use the gradient at x2+y2=r2 with r=24 or 18 as the initial conditions. How do the performances compare (w.r.t. actual p & q)? From the estimated map recover the depth z(x,y). How does the recovered depth compare with the actual depth?

- Add noise to the image and redo the experiment. Compare the MSE as a function of the variances of noise. Repeat the experiment with different ps,qs (See papers for using ps and ps).

- Assume that the exact knowledge of ps,qs is not available i.e. you only know a noisy estimate of ps,qs. Repeat the experiment with varying amount of errors in source localization.